DSC SAN JUAN 24

I was out on the SJ24 today in the warm sunshine. And I thought of  a

2012 Aquavit To Do List:

The Honda motor runs "hull speed " at 1/2 throttle, and sits low in the water. The motor should not be drowned up to the neck and cover. It should be in the water so the bigger fin is just under the surface.  You will have to monitor and re-adjust if 4 people sit on the stern and motor into waves, So ADJUST IT FOR THE LOAD.

Honda Motor needs an Oil Change – Really Read the instructions first.

let me know. If you have problems call my Cell 778-554-7700.

Reverse tends to pop the blade out of the water when pushed past a "fast idle" - so you will learn , or get soaked.

Now the exciting stuff.

The  Lifeline on Port Stern is fractured and replaced by a rope.  Nip over to Granville Island and borrow the clamp and to Steveston Marine, and buy the appropriate line, and Swages, then measure it while on the boat and clamp in the right places. Include a stiff rubber tube around the line as it goes through the stancheon holes. Make the gate on the Starboard as well as port.

The Port rail needs be checked for possible new bead of Sealant.

The  mounted GPS is cracked, I am still looking for a replacement.

A gift Auto Helm is available for installation but needs custom engineering work. Electrical line, fuse and switch, and the angles measured out for position.

The hold , the lockers, with gas tanks, old oil spills, sludge and all that Anchor chain need to be sorted out and Someone to wash away the gummy bears.  Some new Buckets and brushes could be needed. Move the new black fenders to the Club Room.

Suggestions to put the Anchor on the Bow and feed the Chain into the front locker ?  There must be another SJ-24 that had it done properly. I have the Hawser Cover in a drawer.

The Air Vent just aft O the Mast has been kicked loose or fractured. Can it be remounted, or glued or go buy a new one for $80.oo. Or put the margarine tub back on it.

The Stanchions need be checked for leaks underneath and the lazaret covers too.

. The sink is really really dirty. Check what is in the silverware drawer and are their dishes in the cupboard ? Do we actually use cooking and dishes ?

The electric Water pump is not wired in, needs to be soldered connections. Mayby it is time to replace the pump-faucet.

How many Life jackets and are they clean enough to put on ?

Clean the decks, Could be pressure washed, but a brush & Marine Clean work as well.  We could paint the decks or at least the CockPit seats. \

The bow cleats and fairleads need those support blocks checked.

On the right of the Hatchway, cabin top is  the remainder of a screw hole set that used to have a Halyard jam cleat. Tape is covering it.  It was never thru-deck bolted so it pulled off around 2004. This could be sealed providing the Balsa Core has dried out this summer.  No more water is to be allowed into this spot as it compromises the cabin roof Balsa Core.

And of course the sea life has become accustomed to living on the hull. Likely due for a  night on the Tidal Grid, to -wash-scrub-paint" . It takes 2-3 volunteers who will be important, in accomplishing a time limited - heavy exercise job. $60.oo for the 24 hour use.

A new scrap of cut -to fit- carpet would feel good on the overnight trips.

Sails may need review, and even to repair.  The 2 main Sails are of different shape. We have some donations and one might fit. But someone needs to take on the job of unfolding and measuring and installing a new set of “Sew On” sliders- if we find a good freebie. Might need an industrial sewing machine for this one.  OR just order a new MainSail from “Far East Sails” only  $1100.oo  OR see if there is a useable one at PopEyes or Steveston used.

Can anyone check all the rigging,  the blocks, halyards, cleats Chainplates.  Are they properly tensioned and the turnbuckles locked.

Are their spare parts in a nice clean “Bosuns Box” and should we upgrade the toolbox ? It is a bit rusty and is there anything useful besides a screwdriver ?

The spinnaker is great but can be dangerous to use. It needs a functioning and proper size pole and a Snuff Sock manufactured. There are some donated sails which could be stitched into a snuff sock. - Instructions are on the internet.

Life Lines that Ken & Brian Nordquist installed in about 2006 are frayed and broken.  Replacing them is: A/ Buy new and get the tool & fittings and build them on the dock.

B/ use old Hobie Cat Shrouds and such and buy swaging nuts and use the clamp tool and buy only the gate clip.  But regardless, find a way to protect the line as it passes the stanchion eyelets.